DEVELOPMENT ACTIVITIES

BARIND MULTIPURPOSE DEVELOPMENT AUTHORITY

RAJSHAHI

December’ 2024

BARIND MULTIPURPOSE DEVELOPMENT AUTHORITY

December’ 2024

Background of BMDA :

The part of greater Rajshahi, Dinajpur, Rangpur and Bogra District of Bangladesh and the Indian territorial Maldah District of West Bengal is geographically identified as Barind Tract. The hard red soil of these areas is very significant in comparison to that of the other parts of the country.

The Rajshahi Barind Tract is located in between 24 degree 23 minute to 25 degree 15 minute north latitude and 88 degree 2 minute to 88 degree 57 minute east longitude. A typical dry climate with comparatively high temperature prevails in Barind area except for the wet season beginning from mid June to October. Rainfall in the area varies from about 1500 mm to 2000 mm. Temperature ranges from 4 degree Celsius to 44 degree Celsius. The total cultivable area being 1.44 million acres, out of which 34% is loamy, 10% is Sandy, 49% is clayed and the rest of 7% is others. Out of the total cultivable land, 84% are single cropped, 13% are double cropped and the rest are triple cropped. The cropping intensity was 117%.

The Rajshahi Barind Tract was much flourished and the Socio-economic activities were much better in comparison to that of the other part of the country before 1947. In that time the area was well communicated to Kolkata, the capital city of west Bengal, now in India. Trade and commerce was much dependent on Kolkata and this Region was the hinterland of Kolkata port.

After partition of India communication, trade and commerce of this region with Kolkata was ceased totally and all the socio-economic activities almost came to a hault. As there was no well road linkage to the other part of the country, even to capital city Dhaka; no effort could make any development or any upliftment of the socio-economic structure of this region. As days passed by, this tract of land had fallen behind in all phases of development and economic activities could not be generated much. Further, the latest development in industrialization within the country could not make any headway in this Region. As a result the standard of living of the people of this region came to a lower level compared to that of the other parts of the country.

In 1985 the Government approved a project named “Barind Integrated Area Development Project” under the Bangladesh Agricultural Development Corporation. In 1990 the project was declared complete using only 26% of the fund allocated. On completion of the project a review was conducted and to speed up the development project implementation of the Barind area a separate authority was created named Barind Multipurpose Development Authority (BMDA) on 15 th January 1992 under the ministry of Agriculture. Till Now The authority has so far implemented 46 (Forty-six) development projects.

Jurisdiction of BMDA :

According to the resolution, forming BMDA, firstly all the highland which are normally free from flood and draught prone area of Rajshahi Division are under the Jurisdiction of BMDA. At present the Act of the Authority has passed by the Government on 1st October 2018 named “BMDA Act-2018”. According to the Act, all the districts (Rajshahi, Chapai Nawabganj, Naogaon, Joypurhat, Bogra, Sirajganj, Pabna and Natore) of Rajshahi Division and all the districts (Thakurgaon, Dinajpur, Panchagar, Rangpur, Kurigram, Nilfamari, Lalmonirhat and Gaibandha) of Rangpur Division are under the development Programme of Barind Multipurpose Development Authority (BMDA).

Scope of BMDA :

The Development priorities of the Barind Area particularly concerning the development of Agriculture 	include:-

a)  To ensure the development of ground & surface water resource and its proper use for irrigation.

b)  Agricultural mechanization, seed production & supply and crop diversification.

c)  Afforestation to achieve environmental and ecological balance.

d)  Construction of connecting road in small scale & maintenance for marketing of agricultural product.

e)  Installation of irrigation equipment and supply of pure drinking water to the human habitation.

f)  Complete the contract with the approval of the Government.

g)  Research and training.

h)  Other activities …...

Total Development activities under various projects of BMDA:

| Development activities | Achievement |
| --- | --- |
| Installation of DTW | 15828 |
| Installation of LLP (Solar+Electric) (Nos.) | (420+576) 996 |
| Irrigation Water Distribution System (Km.) | 14604.69 |
| Extension Irrigation Water Distribution System (Km.) | 1883.95 |
| Re-excavation of Canals (km.) | 2594.82 |
| Re-excavation of Derelict Ponds (Nos.) | 4276 |
| Construction of Cross dam (Nos.) | 782 |
| Digging of Dug Well with solar energy (Nos.) | 650 |
| Feeder Road Construction (Km.) | 1144 |
| Drinking water supply from irrigation DTW (Nos.) | 1715 |
| Construction of Foot over Bridge (Nos.) | 51 |
| Construction of Cattle cross culvert (Nos.) | 109 |
| Construction of Light Culvert (Nos.) | 8 |
| Elimination of water logging (Hec.) | 13057 |
| Supply of Hose/Fita Pipe (Meter) | 588100 |
| Afforestation (Lakh) | 270.59 |
| Farmer training (Person) | 156272 |
| Utilization of irrigation equipment in 2023-24 (Nos.) | DTW 15538 + LLP 891 |
| Irrigated area in 2023-24 (Lakh Hec) | Aush 0.811 + Amon 3.596 + Robi 5.945 |
| Irrigation charge realization in 2023-24 (Tk. Lakh) | 25081.10 |
| Utilization of irrigation equipment in 2024-25 (Nos.) | DTW 13824 + LLP 596 |
| Irrigated area in 2024-25 (Lakh Hec) | Aush 0.805 + Amon 3.546 + Robi 2.993 |
| Irrigation charge realization in 2024-25 (Tk. Lakh) | 5189.02 |
| Cropping intensity (in the command area (before project 117%)) | 240% |

ACHIEVEMENTS OF BARIND AUTHORITY AT A GLANCE

#### A. ONGOING PROJECTS

1.	Expansion of Irrigation in Natore District Through Best Uses of Surface Water and Conservation of Rain Water, (July-2019- December'2024). Estimated cost: Orig.: Tk. 17557.52  lakh & Rev.: Tk. 19120.29  lakh.

Objective of the project : a) Increasing surface water reservoirs, conservation water, its use in irrigation works, reduction stress in ground water and enhancement of ground water recharge through re-excavation of derelict canals. b) Make agriculture via drainage on 852 hectares of arable land. c) To increase the use of renewable energy and reduce the pressure on electricity by setting up solar powered LLPs. d) Production of drought tolerant and vibrant consumer crops (onion, garlic, cucumber, eggplant, sweet lava, pumpkin, gossip, melons, vegetables etc.) and limiting the excessive use of ground water by digging dugwell. e) 56688 MT additional crop production in 10505 hectare of land by providing low cost and planned irrigation. f) Creating additional forest resources, increasing the supply of nutrients and supporting the improvement of environment by planting 2.00 lac fruits and medicinal plants. g) Providing training to 800 ideal farmers on modern agriculture, irrigation infrastructure, moderate use of ground and surface water.

(Lakh Taka)

| # Components | DPP Target | Achievement up to December, 2024 | For the year 2024-25 |  |  | Cumulative Expenditure (%) Physical % |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ADP Allocation | Fund released | Expenditure (%) physical % |  |
| Plantation (Fruit, Forest & Medicinal Plants) (Nos.) | 200000 | 200000 | 2630.00 | 1563.95 | 1563.95 (59.47%) 100% | 17894.132 (93.59%) 100% |
| Farmers training (Person) | 800 | 800 |  |  |  |  |
| Construction of foot overbridge (Nos.) | 30 | 30 |  |  |  |  |
| Drainage canal construction (Nos.) | 20 | 20 |  |  |  |  |
| Cattle cross culvert construction (Nos.) | 60 | 60 |  |  |  |  |
| Construction of Hydrolic elevated dam (Nos.) | 3 | 3 |  |  |  |  |
| Construction of uPVC pipeline for LLP (Nos.) | 60 | 60 |  |  |  |  |
| Extension of uPVC pipeline (610 m. each) (Nos./km.) | 170/103.70 | 170/103.70 |  |  |  |  |
| Re-excavation of khal/khari (49493 cum/km) (Km.) | 155 | 155 |  |  |  |  |
| Digging of Dugwell (120-130 ft) with water distribution system (360 m. each) (Nos.) | 60 | 60 |  |  |  |  |
| Construction of uPVC drainage canal for elimination of Water Logging (M.) | 3750 | 3750 |  |  |  |  |

2.	Expansion of Irrigation in Greater Rangpur District Through Best Uses of Surface Water and Conservation of Rain Water (1st Rev.); (Oct.,2019-June,2025). Estimated cost: Tk. 25056.63 lakh Rev.: Tk. 28811.63 Lakh.

Objective of the project : a) Increasing surface water reservoirs, conservation water and its use in irrigation through re-excavation of canals, ponds & beels. b) Make agriculture via drainage on 1000 hectares of arable land. c) 112240MT additional crop production in 13504 hectare of land by providing low cost and planned irrigation. d) To increase the use of surface water for irrigation through 213 nos. of LLP's. e) To increase the use of renewable energy and reduce the pressure on electricity by setting up solar powered LLP's. f) Production of low water consuming crops by digging 50 nos. solar powered dugwells. g) Creating additional forest resources, increasing the supply of nutrients and supporting the improvement of environment by planting 2.30 lac fruits, forests and medicinal plants.

(Lakh Taka)

| # Components | DPP Target | Achievement up to December, 2024 | For the year 2024-25 |  |  | Cumulative Expenditure (%) Physical % |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ADP Allocation | Fund released | Expenditure (%) physical% |  |
| Plantation (Fruit, Forest & Medicinal) (Nos.) | 230000 | 230000 | 4551.00 | 2273.45 | 1762.80 (38.73%) 40.12% | 25789.70 (89.51%) 93.32% |
| Farmers training (Person) | 1125 | 1125 |  |  |  |  |
| Construction of Divisional Office Building (Nos.) | 1 | 1 |  |  |  |  |
| Construction of Regional Office Building (Nos.) | 1 | 1 |  |  |  |  |
| Construction of Zonal Office Building (Nos.) | 8 | 8 |  |  |  |  |
| Re-excavation of khal/khari (km.) | 230 | 215 |  |  |  |  |
| Re-excavation of Beel (Nos.) | 10 | 10 |  |  |  |  |
| Re-excavation of Pond (Nos.) | 99 | 83 |  |  |  |  |
| Construction of Sub-mergedweir (Nos.) | 10 | 8 |  |  |  |  |
| Construction of Footoverbridge (Nos.) | 20 | 14 |  |  |  |  |
| Installation of Solar LLP(Nos.) | 30 | 30 |  |  |  |  |
| Installation of Ellectric LLP(Nos.) | 183 | 180 |  |  |  |  |
| Digging of DugWell with pipeline (Nos.) | 50 | 50 |  |  |  |  |
| Construction of uPVC pipeline (Km.) (1000 m./137 nos. & 1655 m./76 nos.) | 262.78 | 231.33 |  |  |  |  |
| Drainage canal & cattle cross culvert Cons. (Nos.) | 50 | 50 |  |  |  |  |
| Installation & commissioning forced mood pump | 20 | 20 |  |  |  |  |

3.	Enhancement of Irrigation Throgh Surface Water Augmentation in Greater Dinajpur & Joypurhat District.; (October-2020- December'2025). Estimated cost: Org: Tk. 25114.79 lakh; Rev. Tk. 28867.79 Lakh.

Objective of the project : 1) Bringing non-irrigated land of the project area under irrigation. 2) Expansion of irrigation and removal of water logging by creating water reservoir & conserving rain water. For this- a) Re-excavation of 200 km Khals & 100 nos. water body and construction of 37 nos. submersedweir b) Installation of 155 nos. solar/electric LLPs and other facilities in the project area. c) To produce 2.32 lakh MT crop every year by irrigating 26506 hectares of land using 155 nos. of solar/electric LLP. d) Bringing 306 hectares submersed land under agricultural production by removing water logging. e) Ensuring low water consuming crop /vegetable production by excavating 60 nos. of Dugwells. f) Reduce the use of electricity for irrigation through the use of renewable solar energy. g) To create additional forest resources, meet nutritional needs and help in environmental development by planting 2 lakh trees. h) To create opportunities for farmer training and self-employment in the project area.

(Lakh Taka)

| # Components | DPP Target | Achievement up to December, 2024 | For the year 2024-25 |  |  | Cumulative Expenditure (%) Physical % |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ADP Allocation | Fund released | Expenditure (%) physical% |  |
| Plantation (Fruit, Forest & Medicinal) (Nos.) | 200000 | 200000 | 5500.00 | 2749.22 | 2114.25 (38.44%) 45.39% | 24825.74 (86.00%) 87.14% |
| Farmer Training (Person) | 1500 | 1500 |  |  |  |  |
| Construction of Zonal office (Nos.) | 4 | 4 |  |  |  |  |
| Construction of Store Cum Zonal office (Nos.) | 6 | 5 |  |  |  |  |
| Construction/repair of office boundary (Nos.) | 5 | 4 |  |  |  |  |
| Digging of DugWell (Solar and pipeline) (Nos.) | 60 | 40 |  |  |  |  |
| Re-excavation of khal/khari (km.) | 200 | 170 |  |  |  |  |
| Re-excavation of Water Body (Nos.) | 60 | 58 |  |  |  |  |
| Construction of Sub-merged weir (Nos.) | 37 | 23 |  |  |  |  |
| Construction of uPVC Pipeline (250 mm dia) (Nos./km.) | 155/232.50 | 127/190.50 |  |  |  |  |
| Extension of uPVC pipeline Each-590m (Nos./km.) | 1500/885 | 1135/669.65 |  |  |  |  |
| Installation of Solar LLP (Nos.) | 125 | 110 |  |  |  |  |
| Installation of Electric LLP (Nos.) | 30 | 30 |  |  |  |  |
| Water drainage Pipe Line (625mm dia) (Nos.) | 2.50 | 2.50 |  |  |  |  |

4.	Rehabilitation of Irrigation Infrastructure Project; (January' 2022 - December' 2026).

Estimated cost: Tk. 32298.71 lakh.

Objective of the project : a) To maintain irrigation facilities by rehabilitating old 6654 nos. Deep tubewells; b) To produce 3.26 lakh M: tonnes of crops annually by ensuring irrigation of 40740 hectares of land; c) To prevent of wastage of irrigation water through repair/renovation of irrigation canals; d) Increasing the efficiency of irrigation equipment; e) To improve the quality of life of about 54 thousand people in the project area through agricultural work.

(Lakh Taka)

| # Components | DPP Target | Achievement up to December, 2024 | For the year 2024-25 |  |  | Cumulative Expenditure (%) Physical % |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ADP Allocation | Fund released | Expenditure (%) physical% |  |
| Rehabilitation of old deep wells (Nos.) | 1358 | 825 | 8500.00 | 4249.00 | 4222.50 (49.68%) 57.70% | 20859.27 (68.58%) 76.93% |
| Reconstruction of old unusable pump house (Nos.) | 2309 | 1685 |  |  |  |  |
| Repair of old pump house (Nos.) | 2987 | 2485 |  |  |  |  |
| Reconnect the Electric line (Nos.) | 1358 | 825 |  |  |  |  |
| Installation of riser valves and repair of outlets (Nos.) | 36789 | 26150 |  |  |  |  |
| Electrical line repair (Nos.) | 300 | 249 |  |  |  |  |

5.	Enhancoment of Command Area and Irrigation Efficiency Through Extension of Subsurface Irrigation Channel and Piloting of Drip Irrigation System; (April,2022-March,2026). Estimated cost: Tk. 32901.40 lakh.

Objective of the project : a) To ensure irrigation facilities of 45000 hectares of land by extension of uPVC pipeline through 4500 of ​​irrigation equipments. b) To prevent of wastage of precious irrigation water through controlled and optimal use of irrigation water. c) To produce 3.60 lakh ton of crops through increasing the command area of ​​irrigation equipments and well controlled irrigation management. d) Providing training to 4,000 farmers for disseminating the latest improved agricultural technologies and techniques among the farmers with a view to expanding the state-of-the-art agricultural technology. e) Experimental introduction of drip irrigation system to increase irrigation efficiency. f) To create the employment opportunities of marginal farmers and daily labourers.

(Lakh Taka)

| # Components | DPP Target | Achievement up to December, 2024 | For the year 2024-25 |  |  | Cumulative Expenditure (%) Physical % |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ADP Allocation | Fund released | Expenditure (%) physical% |  |
| Procurement of uPVC pipe & fittings (250 mm: dia) for construction of underground irrigation canal (M.) | 2322000 | 2321200 | 11000.00 | 5499.50 | 4367.88 (39.71%) 72.41% | 24877.098 (7.61%) 88.94% |
| Procurement of Riser valves (Nos.) | 36000 | 31950 |  |  |  |  |
| Procurement of Ribbon pipes and fittings (M.) | 450000 | 308000 |  |  |  |  |
| Construction of Underground uPVC Pipe Line (M.) | 2250000 | 1598900 |  |  |  |  |
| Drip irrigation system (Nos.) | 10 | 5 |  |  |  |  |
| Farmer’s Training (Persn) | 4000 | 3000 |  |  |  |  |

6.	Extension of Irrigation in Barind Area Through Conservation of Water in Canal-2nd Phase; (October,2022-June'2027). Estimated cost: Tk. 24940.00 lakh.

Objective of the project : a) To increase the use of surface water from 10% to 12% in the project area for irrigation. B) To increase the irrigation facilities by using surface water in 3490ha (approx) land. c) Environmental improvement through plantation besides canals and ponds.

(Lakh Taka)

| # Components | DPP Target | Achievement up to December, 2024 | For the year 2024-25 |  |  | Cumulative Expenditure (%) Physical % |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ADP Allocation | Fund released | Expenditure (%) physical% |  |
| Purchase of pontoons and accessories | 4 | 2 | 8000.00 | 9999.44 | 2947.33 (36.84%) 52.12% | 11473.964 (46.01%) 53.10% |
| Pontoon repairs and ancillary works | 2 | 2 |  |  |  |  |
| Re-excavation of Khas khal/khari (km.) | 38.25 | 29 |  |  |  |  |
| Re-excavation of the pond (No.) | 1 | 1 |  |  |  |  |
| Re-excavation of the beel (Nos.) | 2 |  |  |  |  |  |
| Construction of sub-merged wire. (Nos.) | 7 | 3 |  |  |  |  |
| Construction of sub-mergedwire with footover bridge (Nos.) | 6 | 2 |  |  |  |  |
| Construction of pipe holding structures (No.) | 1 |  |  |  |  |  |
| Construction of box culverts across highways (No.) | 1 |  |  |  |  |  |
| Dismantling 250mm. Dia pipe line (km.) | 42.00 | 42.00 |  |  |  |  |
| Dismantling 350mm. Dia pipe line (km.) | 11.10 | 11.10 |  |  |  |  |
| Road demolition and reconstruction, bitumen carpeting (km.) | 2.30 | 0.70 |  |  |  |  |
| Construction of 250mm. Dia uPVC pipe line (km.) | 132 | 40 |  |  |  |  |
| Construction of 250mm. dia underground MS pipe lines (km.) | 16 | 6 |  |  |  |  |
| Construction of 450/500 mm. dia HDPE pipe line (km.) | 38.70 | 23 |  |  |  |  |
| Construction of 450/500mm. dia underground MS pipe lines (km.) | 34 | 8 |  |  |  |  |
| Construction of river banks, pump landing station, pump house etc (Nos.) | 8 | 6 |  |  |  |  |
| Construction of RCC structure at water fall site (Nos.) | 7 | 5 |  |  |  |  |
| Construction of electric lines (km.) | 13 | 8 |  |  |  |  |
| Installation and commissioning of transformers (Set.) | 16 | 12 |  |  |  |  |
| Installation and commissioning of pumps (Set) | 26 | 17 |  |  |  |  |

7.	Program on Agricultural and Rural Transformation for Nutrition, Entrepreneurship and Resilience (PARTNER) (July' 2023 - June' 2028). Estimated cost: Tk. 11092.00 Lakh (GOB: Tk.709.00 Lakh; DPA: Tk. 10383.00 Lakh);

Objective of the project : 1. Fruits and Vegetables with GAP certification - Increase area under cultivation. 2. Expansion of new areas under high yielding rice varieties. 3. Increasing the area under cultivation of non-rice grains, pulses, oilseeds and horticulture crops. 4. Bringing new land under cultivation under improved and efficient irrigation technology. 5. Ensuring proper administration and availability of agricultural services (such as extension assistance, fertilizer seed and other input assistance and agricultural credit assistance) through "Farmer Smart Card" system 6. Accreditation and promotion of seed certification and food safety testing 7. Providing training to youth and women entrepreneurs in commercial agriculture, agribusiness, agricultural innovation and agricultural services 8. Development of an operational evaluation system for NARS institutes and extension systems along with enhancement of research activities for new technologies and innovations to enhance the capacity of institutions 9. Establishment and operationalization of promotional organizations in at least five value chains for selected agricultural products. 10. Quality development of integrated digital agricultural information services including agricultural statistics and international markets.

(Lakh Taka)

| # Components | DPP Target | Achievement up to December, 2024 | For the year 2024-25 |  |  | Cumulative Expenditure (%) Physical % |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ADP Allocation | Fund released | Expenditure (%) physical% |  |
| Installation of Solar system for existing DTW. | 80 | 23 | 4776.00 | 2353.48 | 840.93 (17.61%) 1.51% | 4522.14 (40.77%) 34.73% |
| Installation of Solar LLP | 50 | 30 |  |  |  |  |
| Installation of New Solar DTW in Char land | 50 | 20 |  |  |  |  |
| Construction of buried pipe line for LLP (each 610 m) with supply of 250 mm dia uPVC pipes, fittings and riser valves etc. (package) | 50 | 10 |  |  |  |  |
| Construction of buried pipe line for DTW in Char land ( each 610 m) with supply of 250 mm dia uPVC pipes, fittings and riser valves etc. (package) | 50 | 25 |  |  |  |  |
| Installation of Drip Irrigation System | 10 |  |  |  |  |  |
| Installation of Sprinkler Irrigation System | 10 |  |  |  |  |  |
| Installation of Sensor based irrigation System | 1 |  |  |  |  |  |

8.	Popularize of High Value Unconventional Fruit & Medicinal Crop in Barind Area Project;

(July-2020- June'2026). Estimated cost: Org.: Tk. 1733.82 lakh; Rev.: Tk. 2906.63 lakh.

Objective of the project : a) For the shake establishing high value non-conventional fruits and medicinal crops garden in farmers' own land or homestead area disbute saplings free of cost. b) Establishment of demonstration farms of high value non-conventional fruits and medicinal crops in the Barind area.. c) Provide training to farmers / officers-staffs on production of high value fruit and medicinal saplings, planting etc.

(Lakh Taka)

| # Components | DPP Target | Achievement up to December, 2024 | For the year 2024-25 |  |  | Cumulative Expenditure (%) Physical % |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ADP Allocation | Fund released | Expenditure (%) physical% |  |
| Farmer's Training (Person) | 1500 | 1050 | 675.00 | 336.79 | 319.95 (47.40%) 55.38% | 2007.46 (69.06%) 72.44% |
| Officer/Employee Training (Person) | 270 | 240 |  |  |  |  |
| Demonstration farming (Nos.) | 100 | 77 |  |  |  |  |
| Purchase of sapling (Nos.) | 416800 | 331800 |  |  |  |  |
| Purchase of seeds (Nos.) | 5050 | 4080 |  |  |  |  |

9.	Project for Capacity Building Acceleration Including Infrastructure Development of Barind Multipurpose Development Authority. (October,2023- June,2026). Estimated cost: Tk. 4055.00 Lakh (GOB);

Objective of the project : 1. Construction of modern office buildings for development of agricultural work, monitoring, increasing mobility and proper management of proper supervision. 2.To create a better working environment for performing fair work by avoiding obsolete, unsanitary, hazardous. 3. To provide modern and advanced training on agriculture/irrigation including office management, procurement policy, modern irrigation management, SDG, APA, Delta Plan etc. for capacity building of authority officers/parties.

(Lakh Taka)

| # Components | DPP Target | Achievement up to December, 2024 | For the year 2024-25 |  |  | Cumulative Expenditure (%) Physical % |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ADP Allocation | Fund released | Expenditure (%) physical% |  |
| Construction of Head Quarter office building (Sqm.) | 7680 |  | 2235.00 | 558.55 | 120.75 (5.40%) 7.05% | 195.75 (4.83%) 8.57% |
| Construction of Zone office Building (1 no.) (Sqm.) | 335 | 288 |  |  |  |  |

10.	Supply of Padma River’s  water to Barind area for extension of irrigation by double lifting system. (September' 2023 - June' 2027). Estimated cost: Tk. 54805.68 Lakh (GOB);

Objective of the project : 1. Optimum use of surface water in the project area through development of modern and sustainable irrigation system, prevention of wastage of irrigation water , increase in recharge of groundwater and production of additional crops by expanding irrigation. 2. 18 km away from Padma river and 35 meters (115 ft) high in Barendra area, Padma river water supply for irrigation system in the pre-project area has increased 10250 hectares of one crop land under irrigation to three crop lands and increased about 1.4 lakh MT of crops every years. 3. To increase the current irrigated area through surface water in the project area from 2012 ha or 2.56% to 12262 ha or 15.60% and reduce the pressure on groundwater. 4.Increasing the water holding capacity of the canal by re-excavating 120 km of maza Canal in the project area and providing year-round irrigation to the command areas on both sides of the canal by channeling the water of the Padma River into the said canals 5. Increasing groundwater recharge and control groundwater table subsidence through use of surface water in the project area. 6. In the project area there is an Excavated canal crossing both the yellow and medicinal plants to combat climate change and improve the environment. 7. Providing training to 600 ideal farmers on modern agriculture, irrigation infrastructure, moderate use of ground and surface water.

(Lakh Taka)

| # Components | DPP Target | Achievement up to December, 2024 | For the year 2024-25 |  |  | Cumulative Expenditure (%) Physical % |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ADP Allocation | Fund released | Expenditure (%) physical% |  |
| Construction of Box Culvert (for Road crossing) (Nos.) | 10 |  | 14056.00 | 3514.00 | 545.86 (3.88%) 6.73% | 791.20 (1.44%) 1.82% |
| Construction of Sub merged wire (Nos.) | 50 |  |  |  |  |  |
| Intake Pump with Prime Mover (60 Cusec) (No.) | 1 |  |  |  |  |  |
| Centrifugal Booster Pump 60 cusec & 130ft Head with necessary accessories (Nos.) | 2 |  |  |  |  |  |
| Construction of 120 Cusec discharge Booster Pump (centrifugal pump) station with pump base(sqm.) | 1000 |  |  |  |  |  |
| Procurement of 250 mm dia uPVC Pipe (for LLP) (M.) | 160000 |  |  |  |  |  |
| Procurement of Solar cum Electrified LLP & Net Metering with other Irrigation equipment (Nos.) | 160 |  |  |  |  |  |
| construction of LLP & Net Metering with other Irrigation structure (Nos.) | 160 |  |  |  |  |  |
| Procurement of Drip Irrigation equipment (Nos.) | 10 |  |  |  |  |  |
| Installation of Drip irrigation system with associated equipment (Nos.) | 10 |  |  |  |  |  |
| Procurement of 1000 mm dia MS pipe with fittings | 37000 |  |  |  |  |  |
| Installation of 1000 mm dia MS pipe with fittings | 37000 |  |  |  |  |  |
| Re Excavation of khal | 120 | 7 |  |  |  |  |
| Procuring of Electrification materials for construction of electric line | 125 |  |  |  |  |  |
| Procuring of Motor cycle | 6 |  |  |  |  |  |
| Procurement of Tele pre paid meter for LLP | 320 |  |  |  |  |  |
| Installation of Tele pre paid meter for LLP | 160 |  |  |  |  |  |
| Setting up of 20 MVA sub station ( Intake pump=10 MVA, Booster pump 5+5=10 MVA) | 1 |  |  |  |  |  |
| Plantation (Fruit & Medicinal) (Lakh) | 1.99 |  |  |  |  |  |

B. LIST OF COMPLETED PROJECT

| Project Name | Allocation & Expenditure (Lakh Tk.) | Components | Achievement |
| --- | --- | --- | --- |
| 1. Barind Integrated Area Development Project-Phase-I (1985-1990) | 39497.75 & 10411.00 (26.36%) | a) Installation of DTW (nos.) b) Re-excavation of Ponds (nos.) c) Re-excavation of Canals (miles) d) Water Distribution System (nos.) e) Electrification of DTW (nos.) f) Construction of Feeder Road (km.) g) Aforestation (Lakh) | 1512 1170 6.50 26 10 47.98 49.21 |
| 2. Barind Integrated Area Development Project-Phase-II (1990-2001) | 49168.00 & 46512.96 (94.60%) | a) Installation of DTW (nos.) b) Re-excavation of Derelict Ponds (nos.) c) Re-excavation of Canals (km.) d) Construction of Cross dam (nos.) e) Irrigation Water Distribution System (nos.) f) Electrification of DTW (nos.) g) Feeder Road Construction (km.) h) Aforestation (lakh) | 3536 784 40.50 30 2074 3040 65.85 84.21 |
| 3. Fine Rice Production Project in Barind Area (1995-2001) | 197.00 & 137.52 (69.81%) | a) Seed Procurement (MT) b) Seed Distribution (MT.) c) Farmers Training (nos.) d) Demonstration plot (nos.) | 43.70 25.65 2679 142 |
| 4. Rural Drinking Water Supply (PILOT) Project (1995-1997) | 161.50 & 161.50 (100%) | Drinking Water Distribution System Construction (nos.) | 26 |
| 5. Barind Rural Road Development Project-Phase-I (1994-2000) | 12600.00 & 12520.10 (99.37%) | a) Construction of Feeder road (Km) b) Construction of Culvert, Bridge (Nos.) c) Aforestation (Lakh) | 335.38 186 14.15 |
| 6. Barind Bridging Project (2001-2003) | 2133.00 & 2000.00 (93.76%) | a) Re-excavation of pond. (Nos.) b) Re-excavation of canal (Km) c) Construction of Cross Dam (Nos) d) Rehabilitation of DTW (Nos.) d) Sapling (lakh) | 80 12.80 8 130 4.00 |
| 7. Barind Rural Road Development Project-Phase-II (2000-2003) | 1901.00 & 1832.62 (96.40%) | a) Construction of Feeder road (Km.) b) Construction of Culverts (Nos.) e) Aforestation (Lakh) | 37 22 (162M.) 0.10 |
| 8. Intensive Aforestation Project (1997-2004) | 958.40 & 957.56 (99.91%) | a) Aforestation i) Fruit trees (lakh) ii) Forest trees (lakh) | 5.56 21.66 |
| 9. Repair/Rehabilitation of Deep tube well Project in Greater Dinajpur District Under Post-Drought Agricultural Rehabilitation Programmer (2003-2004) | 1087.83 & 1087.83 (100%) | Rehabilitation of DTW (Nos) | 1217 |
| 10. Barind Command Area Development And Training Project of (1996-2006) | 13804.56 & 13804.56 (100%) | a) Water Distribution System (nos.) b) Farmers training (Person) | 2600 30000 |
| 11. Barind DTW Electrification Project (1996-2006) | 14862.70 & 14862.66 (100%) | a) Electrification of DTW (Nos.) b) Electric Pump purchase (Nos.) c) Construction of sub-station (Nos.) d) Supply & Installation of 3/15 KVA 11/0.4 KV distribution sub-station (Nos.) e) Supply & Installation of 3/10 KVA 11/0.4 KV distribution sub-station (Nos.) f) Rehabilitation of 11/0.04 KV, 3/15 & 3/10 KV distribution sub-station (Nos.) | 2203 2203 5 495 1708 330 |
| 12. Barind DTW Installation Project (2001-2006) | 13050.00 & 13050.00 (100%) | a) DTW installation (Nos.) b) Test well sunk (Nos.) | 1650 Nos (2200 cusec) 100 nos |

| Project Name | Allocation & Expenditure (Lakh Tk.) | Components | Achievement |
| --- | --- | --- | --- |
| 13. Bangladesh Fine Rice Production & Marketing Programme (2004-2006) | 250.00 & 98.38 (39.35%) | a) Training (Person) b) Seed supply and procurement (MT). c) Setting of demonstration Plot (Nos). | 1817 40 70 |
| 14. Eco-System Management in Barind Areas (1999-2006) | 754.21 & | a) Aforestation (lakh sapling) b) Re-excavation of pond (Nos.) c) Re-excavation of canal (Km) d) Cross Dam (Nos.) e) Excavation of Mini ditch. (Nos) f) Water sealed latrine (Nos.) g) Pilot-Home lighting (set) h) Pilot-Biogas plant (Nos.) i) Drinking water supply system j) Established of Eco-village (Nos.) k) Mini Eco-Park (Nos.) | 5.55 43 16.50 11 430 96 23 20 12 9 1 |
| 15. Drinking Water Supply from Irrigation Deep Tubewell Project (2002-2007) | 4124.66 & 4124.53 (100%) | a) Water supply installation (Nos.) b) Water supply installation extension (Nos.) | 282 27 |
| 16. Command Area Development & Training Project, Unit-2, Thakurgaon (2003-2008) | 9748.50 & 9700.50 (99.51%) | a) Burried Pipe Line (Nos) b) Water supply installation (Nos.) c) Sprinkler Irrigation system (Nos.) d) Farmers training (Person) | 1217 12 1 12000 |
| 17. Barind Rural Road Development Project, Phase-3 (2003-2008) | 10000.00 & 9999.76 (100%) | a) Construction of Road (Km.) b) Construction of Bridge/Culvert (Nos.) c) Construction of Office Building (Nos.) d) Road/Culvert repair/maintenance (Km.) e) Aforestation (Lakh.) | 237.50 87 3 686 3 |
| 18. Aforestation Program for Ecological Balance in Unit-2 Area of Barind Project (2005-2008) | 867.69 & 858.26 (98.91%) | a) Aforestation : i) Fruit tree (lakh) ii) Forest trees (lakh) b) Nursery establishment (Nos.) c) Training (Person) | 3.00 10.00 13 2030 |
| 19. Deep Tubewell Installation Project, Unit-2, Thakurgaon. (2005-2010). | 15995.00 & 15994.61 (100%) | a) DTW installation (Nos.) b) Electrification of DTW (Nos.) c) Water distribution System (Nos.) d) Training (Person) | 1100 1100 1100 5000 |
| 20. Barind Rain Water Conservation & Irrigation Project. (2004-2010) | 15117.13 & 15116.81 (100%) | a) Re-Excavation of Canals (Km.) b) Re-Excavation of Derelict Ponds (Nos.) c) Construction of X-Dam (Nos.) | 1000 800 555 |
| 21. Smart Card Based Pre-paid Pump Usage & Energy Measuring System Project. (2008-2010) | 1567.31 & 1567.06 (100%) | a) Pre-paid Meter (Nos.) b) Smart Card (Nos.) c) Mobile Vending Unit (Nos.) d) Card reader e) Server with necessary equipment f) Tower Installation/construction | 3000 30000 100 25 4 9 |
| 22. Quality Seed Production Project on Farmer Level (2006-2011) | 1032.40 & 1031.77 (99.94%) | a) Seed production, procurement, processing & marketing (MT) b) Foundation/breeder seed procurement (MT) c) Training (Person) d) Demonstration Plot (nos.) | 2000 65 12600 625 |
| 23. Barind Environmental Balance Restoration Through Afforestation Project (2006-2011) | 2252.43 & 2251.80 (99.97%) | a) Afforestation (i) Fruit (Lakh) (ii) Medicinal & Forest (Lakh) b) Training (Person) c) Nursery establishment (Nos.) | 16.80 18.70 1042 25 |

| Project Name | Allocation & Expenditure (Lakh Tk.) | Components | Achievement |
| --- | --- | --- | --- |
| 24. Capacity Building & Resource Mobilization for Sustainable Land Management (Eco System Management) in Bangladesh (SLMP, Barind Part) (2009-2011) |  | a) Green Manuring (Kg.) b) Compost Heap (Nos.) c) Eco-village (Nos.) d) Plantation (Nos.) e) Water sealed latrine ( nos.) f) Tissue culture lab. (No) g) bio-gas plant (Nos). h) Re-excavation of pond (Nos.) i) Training (Person ) | 1330 20 5 2486 10 1 2 2 75 |
| 25. Barind Integrated Area Development Project-Phase-III (2007-2012) | 28428.00 & 28424.62 (99.99%) | a) Installation of DTW (nos.) b) Water Distribution System (nos.) c) Electrification of DTW (nos.) d) Aforestation (Lakh) | 2000 2000 2000 0.12 |
| 26. Fine Rice Production & Marketing Programe (2009-2012) | 368.57 & 361.71 (98.14%) | a) Seed procurement, processing, preservation & marketing (MT) b) Foundation/breeder seed procurement (MT) c) Training (Person) d) Demonstration Plot (nos.) | 416.82 3.27 16000 390 |
| 27. Drinking Water Supply from Irrigation Deep Tubewell Project (2008-2012) | 9302.76 & 9302.26 (99.99%) | a) Water supply installation (Nos.) b) Water supply installation extension (Nos.) c) Training (Person) | 695 24 1500 |
| 28. Command Area Development & Training Project, Phase-2 (2008-2013) | 16154.23 & 16123.42 (99.80%) | a) Burried Pipe Line (Nos) b) Farmers training (Person) | 2500 23500 |
| 29. Command Area Extension & Development Project (2010-2013) | 5124.59 & 5074.06 (99.01%) | a) Extension of Burried Pipe Line (Nos) b) Rehabilitation of DTW (Nos.) c) Farmers training (Person) | 1000 100 6500 |
| 30. Activating Inoperable DTW for Irrigation Project (2008-2014) | 19913.09 & 19903.32 (99.95%) | a) Activating Inoperable DTW(Nos.) b) Water Distribution System (nos.) c) Electric line construction (Nos.) d) Pre-paid Meter installation (Nos.) e) Hose/Fita Pipe (Km.) | 2415 2415 2113 400 20 |
| 31. Irrigation programme through surface water augmentation (2013-2016) | 4060.94 & 4060.08 (99.98%) | a) Re-excavation of Canal (Km.) b) Construction of X-Dam (Nos.) c) Installation of Pontoon (Nos.) d) Installation of LLP (Nos.) e) Water Distribution System (nos.) f) Hose/Fita Pipe (Km.) | 30 7 2 117 117 10 |
| 32. Smart Card Based Pre-paid Pump Usage & Energy Measuring System Project. (2008-2016) | 1908.82 & 1847.68 (96.80%) | a) Pre-paid Meter (Nos.) b) Smart Card (Nos.) c) Mobile Vending Unit (Nos.) d) Card reader | 4100 82000 160 34 |
| 33. Panchagar, Thakurgaon, Dinajpur and Joypurhat Integrated Area Development Project (2010-2017) | 23225.45 & 23216.35 (99.96%) | a) Installation of DTW (nos.) b) Water Distribution System (nos.) c) Extension of Burried Pipe Line (Nos) d) Electrification of DTW (nos.) e) Re-excavation of Canal (Km.) f) Re-excavation of Ponds (Nos.) g) Construction of X-Dam (Nos.) h) LLP installation with irrigation chanel i) Aforestation (Lakh) j) Farmers training (Person) | 790 790 725 790 112 11 10 80 8.00 10400 |

| Project Name | Allocation & Expenditure (Lakh Tk.) | Components | Achievement |
| --- | --- | --- | --- |
| 34. Deep tubewell Installation project, Phase-ii; (January’2010-June’2018). | 27403.96 & 27400.58 (99.99%) | a) Installation of DTW (nos.) b) Water Distribution System (nos.) c) Extension of Burried Pipe Line (Nos) d) Electrification of DTW (nos.) e) Re-excavation of Canal (Km.) f) Construction of X-Dam (Nos.) g) LLP installation with irrigation chanel h) Construction of Dugwell (Nos.) i) Irrigation chanel for Dugwell (360 M.) (Nos.) j) Aforestation (Lakh) k) Farmers training (Person) | 1115 1115 1115 1115 40.00 2 25 20 20 0.80 3000 |
| 35. Marketing agricultural product through development of rural communication project, (October 2010-June 2018). | 33695.69 & 33683.88 (99.96%) | a) Construction of Feeder Road (Km.) b) Periodic maintenance of constructed road (Km.) c) Construction of culvert (Nos.) d) Aforestation (Lakh) i) Forest 3.14 & ii) Fruits 0.52 | 420 744 108 (382 M.) 3.14 & 0.52 |
| 36. Barind rain water conservation & irrigation project, Phase-ii; (March-2011-June 2018) | 19999.57 & 19995.87 (99.98%) | a) Re-excavation of Canal (Km.) b) Re-excavation of pond (Nos.) c) Re-excavation of Dighi (nos.) d) Construction of X-Dam (Nos.) e) Construction of Dugwell (Nos.) f) LLP installation with irrigation chanel g) Soler irrigation system (Nos.) h) Construction of rubber dam (no.) i) Construction of rechargewell (nos.) j) Aforestation (Lakh) i) Forest & ii) Fruits | 600 200 6 97 110 69 46 1 (62 M.) 5 6 & 2 |
| 37. Rehabilitation of old Deep tubewell in Rajshahi, Chapai-nawabgonj & Naogaon districts; (February’2014-June’2018). | 7724.90 & 7622.57 (98.68%) | a) Rehabilitation of old DTW (nos.) b) Installation of ground water monitoring Data logger (nos.) c) Construction of electric & buried pipe line (Nos.) | 600 25 400 |
| 38. Extension of irrigation in Barind area through conservation of water in cannel; (January’2015-June’2018) | 11613.00 & 11612.22 (99.99%) | a) Re-excavation of Canal (Km.) b) Re-excavation of pond (nos.) c) Construction of X-Dam (nos.) d) Construction of pipe line (M.) e) LLP installation (nos.) f) Installation of solar pump (15 hp) (nos.) g) Aforestation (Lakh) | 38.85 7 16 181000 57 106 0.80 |
| 39. Drinking Piped Water Supply from Irrigation Deep Tubewells, Phase-iii; (July’2013-June’2018) | 11045.00 & 10698.595 (96.86%) | a) Water supply installation (Nos.) b) Water supply installation extension (Nos.) | 550 50 |
| 40. Enhancement of Irrigation Efficiency Through Construction of Sub-surface Irrigation Channel; (July2015-June2019). | 13616.20 13510.52 (99.22%) | a) Construction of underground irrigation channel ( km.); b) Farmers training (Person) | 1080 3000 |
| 41. Extension of irrigation facilities by increasing availability of surface water and removing water logging in Naogaon District; (July 2015-June 2019). | 7912.50 & 7894.60 (99.77%) | a) Re-excavation of Khal/Khari (Km.) b) Re-excavation of Water body (Acre.) c) Construction of Submerged weir with foot bridge (Nos.) | 92.20 14 13 |

| Project Name | Allocation & Expenditure (Lakh Tk.) | Components | Achievement |
| --- | --- | --- | --- |
| 42. Quality Seed Production, Distribution & Farmers’ Training Project for crop production (1st Revised). (July 2015-June 2020) | 1046.23 & 1045.00 (99.88%) | a) Seed procurement, processing, preservation & marketing (MT) b) Farmer Training (Person) c) Officer Training (Person) d) Demonstration Plot (nos.) e) Field Day (Nos.) | 3000 5000 700 400 100 |
| 43. Excavation of Dug-well in Barind Area for cultivation of crops by soft Irrigation (Revised-1); (July 2016-June 2021). | 5348.38 & 5347.756 (99.88%) | a) Excavation of dug-well (Nos.) b) Solar Panel Installation (Nos.) c) 1.5 Inch Dia uPvc Pipe Line construction (360 meter each) (Nos.) | 420 420 420 |
| 44. Extension of irrigation facilities by augmentation of surface water and mitigation of water logging in Bagha, Charghat & Paba Upazillas of Rajshahi District; (October 2015-June 2021). | 2987.06 & 2985.26 (99.94%) | a) RCC Retaining wall b) RCC Slab c) Footover bridge with gatevalve d) Heavy Culvert with gatevalve e) Light Culvert f) Structure for protection of river bank g) Sluice gate h) Re-excavation of Khas Khal/Khari i) Digging of Dug well j) Water distribution system for Dug well k) Water distribution system for LLP | 4035 100 7 2 8 1 1 21.50 20 20 10 |
| 45. Minor irrigation through pond re-excavation and surface water augmentation; (July,2019 - June,2024). | 14687.50 & 14635.25 (99.65%) | a) Re-excavation of pond (Nos.) b) Re-excavation of dighi (Nos.) c) Repair of Zonal Office building. (Nos.) d) Construction of uPVC pipeline (Nos.) e) Plantation (Fruit, Forest & Medicinal) (Nos.) f) Construction of Pond Ghat/Steer (Nos.) | 1025 10 10 105 205000 300 |
| 46. Feasibility Study on "Conservation of Ground Water and Raising Its Use Efficiency and Productivity in Irrigated Agriculture in Bangladesh" (BMDA Part); (July' 2020 - June' 2024.). | 43.92 & 42.292 (96.29%) | a) Purchase of UPVC pipes with necessary equipment and installation of pipelines (Meter) b) Outlet structure construction work (Nos.) c) User card(Nos.) | 1200 8 420 |

OUTPUT

| Development Activities | Before Project | After Project (Rajshahi Barind) |
| --- | --- | --- |
| Irrigated area (lakh hectare) Boro | 0.52 | 3.36 |
| Percentage of Irrigated area (as per cultivated area) | 13% | 57.57% |
| Cropping intensity (in the command area) | 117% | 240% |
| Yearly benefited farmer family (lake) | 1.10 | 5.46 |
| Yearly production crop (lake tons.) | 7.56 | 34.32 |
| Yearly additional crop yield (lake tons.) | - | 26.76 |
| Value of additional crop (Crore taka) | - | 4348.50 |
| Yearly average rainfall (inch) | 54.99 | 64.00 |

Potential of Rajshahi Barind area if all cultivable land

brought under irrigation

| (a) | Total cultivable land in the project area | 5.83 lakh ha. |
| --- | --- | --- |
| (b) | Present production (M.T) (non irrigated) | 34.32 lakh 26.76 lakh |
|  | => Yearly additional production in irrigated area (M.T) | 61.08 lakh |
|  | If all 5.83 lakh ha. cultivable land comes under irrigation, production will be | Total 68.90 lakh M.T. |
| (c) | => Expected Yearly additional production will be | 24.48 lakh M.T |
| (d) | => Value of additional yield per year | 4348.50 Crozes TK. |